# YOKO 1/3“ Hi-End DSP COLOR CCD CAMERA

#### MODEL: RYK-211HB & RYK-212HB                                         2008/09/09

RYK-411HB & RYK-412HB

#### FEATURES

- Color camera with SONY 1/3“ Super HAD CCD sensor in metal case.

- Color turns into B/W when low illumination.

- Employs Digital Signal Processor (DSP) chip-set for image control.

- Pixel number : NTSC=270K / PAL=320K (Normal Resolution )

NTSC=380K / PAL=440K (High Resolution )

- High sensitivity, low smear, high anti-blooming, and high S/N ratio.

- Powerful functions: Auto Electronic Shutter (AES), Auto Iris(AI), Auto Gain Control (AGC), Auto White Balance (AWB), Manual White Balance (MWB), Back Light Compensation (BLC) and Flickerless mode (FL).

- Supports Auto Iris Lens: both Video-Drive & DC-Drive lens.

- Supports both C and CS mount lens.

- Built-in anti-alasing and IR-cut optical filter.

#### SPECIFICATIONS (Note: Design and specifications are subject to change without prior notice.)

| Model No. | DC |  | RYK-211HB | RYK-212HB |
| --- | --- | --- | --- | --- |
|  | AC |  | RYK-411HB | RYK-412HB |
| Image Device |  |  | 1/3“ Color Sony Super HAD | 1/3“ Color Sony Super HAD (Hi-Res.) |
| Picture Elements |  |  | NTSC: 510*492, PAL: 500*582 | NTSC: 768*494, PAL: 752*582 |
| Resolution |  |  | 380 TVL | 470 TVL |
| Min. Illumination |  |  | 0.1 Lux @ F1.2 | 0.2 Lux @ F1.2 |
| S/N Ratio |  |  | More than 48 dB |  |
| Electronic Shutter |  |  | NTSC:1/60~1/100,000, PAL:1/50~1/110,000 |  |
| Flickerless Mode |  |  | On / Off (NTSC:1/100, PAL:1/120) |  |
| Gamma |  |  | 0.45 |  |
| Color turns into B/W (Approx.) |  |  | Illumination < 1~3 Lux @ F1.2 |  |
| Iris Control |  |  | Video / DC Drive |  |
| Lens Mount |  |  | C / CS |  |
| White Balance |  |  | Auto / Manual |  |
| Gain Control |  |  | Auto |  |
| Back Light Comp. |  |  | ON/OFF |  |
| Sync. System |  |  | Internal |  |
| Video Output |  |  | 1 Vp-p / 75 Ohms |  |
| Power Supply |  |  | DC type : DC 12V±10﹪ AC type : AC24V, AC100~240V, DC12V/AC24V Dual (option) |  |
| Power Consumption |  | DC | 120mA |  |
|  |  | AC | 2.8W |  |
| Operating Temp. |  |  | -10℃ to 50℃ |  |