--------------- VP300 MainBoard specification ---------------

Model No. VP-300

|  | Item | Description | Remark |
| --- | --- | --- | --- |
|  | Main Chip |  |  |
|  |  | Intel Celeron (Socket 370) | CPU 466/433/400/366/333 MHz |
|  |  | Intel810 chipsets | Main Board |
|  |  | Bt878 | Video Capture |
|  |  | CH7007 | TV Out |
|  |  | Realtek 8139 | Ethernet Controller |
|  |  | Super I/O IT8712 | ITE |
|  |  | ALS4000 | Audio Solution |
|  | Peripheral I/O |  |  |
|  |  | Compact Flash 2MB to 4MB | Vendor? |
|  |  | Watchdog Timer 1 to 64 seconds | Solution? |
|  |  | IDE Interface x 2 |  |
|  |  | FDD Interface x 1 |  |
|  |  | Parallel Port x 1 | Internal |
|  |  | Serial Port RS232x1 and 232/422/485 x1 | Jumper Switching 232/422/485 |
|  |  | IrDA? |  |
|  |  | PS/2 MS | Internal |
|  |  | PS/2 KB | Internal |
|  |  | Connector PCI Slot x1 | Expansion Bus |
|  |  | GPIO Port 8 IN and 8 Out | 4 on main-board, 4 on daughter-board |
|  |  | Remote Control | MPU-401 I/F |
|  |  | Remote Control PAD | ? Keys |
|  |  | LCD Display (16 x1 Character) |  |
|  |  | System Default Setting | Tact button |
|  |  | M/B |  |
|  | Chipset | 82810 or 82810-DC100(W/Display Cache) | Host Controller |
|  |  | 82801AB (Ultra ATA/33) or 82801AA (66) | I/O Controller Hub |
|  |  | 82802 Firmware Hub (FWH) |  |
|  |  | External Clock Speed | 66 /100 /133 |
|  |  | L2 Cache | 128K |
|  |  | DRAM (DIMM 168pin) | DIMM 168pin x1 (32MB, 64MB, 128MB) |
|  |  | BIOS | Award Flash PnP |
|  |  | Reset Switch Type | 2 pin header |
|  |  | Buzzer built-in |  |
|  |  | Battery Type |  |
|  |  | PROM Type |  |
|  | Video |  |  |
|  |  | BT878 (Quad Function by S/W) | Video Controller |
|  |  | Video Input 4h or 8h | 4 on main-board, 4 on daughter-board, BNC connector |
|  |  | 75 ohm terminator | 4 on main-board, 4 on daughter-board, DIP switch |
|  |  | TV-Video Output (Composite) | RCA connector |
|  |  | Video Input Multiple Type | Analog switch |
|  |  | VGA Connector | H-D D-sub Straight Type 15P, Internal |
|  | Audio |  |  |
|  |  | Audio Input 4h or 8h (Line-in) | 4 on main-board, 4 on daughter-board, 8-pin Female Mini-DIN connector |
|  |  | Audio Output 4h or 8h (Earphone-out) | 4 on main-board, 4 on daughter-board 8-pin Female Mini-DIN connector |
|  |  | TV-Audio Output | RCA connector |
|  | GPIO |  |  |
|  |  | Trigger Mode |  |
|  |  | Protection Method |  |
|  |  | State detect Method |  |
|  |  | Input & Output Spec. | Input: Photo-couple Output: 24V/200mA Relay Dry contact |
|  |  | 4 I/O or 8 I/O | 4 on main-board, 4 on daughter-board |
|  |  | Connector Type | 0.25pitch screw terminal block |
|  | Power |  |  |
|  |  | Supply Voltage | 5Vx10A; 12Vx1A |
|  |  | Internal Switch Power Supply | Open Frame |
|  |  | Power Connector Type and Switch Type | P8,P9 type |
|  |  | Power Pin Number | 12 |
|  | LAN |  |  |
|  |  | Connector Type | RJ45 |
|  | PCB |  |  |
|  |  | Dimension (Little Board) | 267 x 160 (mm) |
|  |  | PCB Layer Number | 4 |
|  | System |  |  |
|  |  | Case Dimension | 381 x 164 x 41(mm) |
|  |  | Fan Location |  |